PM2.5 MULTIMODAL DEEP LEARNING

6 Monitoring Stations — Hanoi, Vietnam

Document 1 of 3

Research Data Summary

Data Sources · Collection · Preprocessing · Splits

Research Period: January 2024 – May 2026

# 1. Project Overview

This research develops a multimodal deep-learning system to predict daily PM2.5 concentrations at 6 ground monitoring stations in Hanoi, Vietnam. Three independent data sources are fused: ground-station air quality measurements (OpenAQ), atmospheric reanalysis weather data (Open-Meteo), and multispectral satellite observations (Sentinel-5P / Sentinel-2).

Research period: 29 January 2024 → 15 May 2026  |  6 stations  |  ~2.5 years of daily observations

# 2. Monitoring Stations (OpenAQ)

Six IQAir monitoring stations operated continuously in Hanoi. Station metadata was sourced from the OpenAQ v3 API and stored in Stations.xlsx.

| Station ID | Name | District |
| --- | --- | --- |
| 2161306 | Minh Khai – Bắc Từ Liêm | Bắc Từ Liêm |
| 2161292 | Số 46 phố Lưu Quang Vũ | Thanh Xuân |
| 4946812 | Công viên Nhân Chính – Khuất Duy Tiến | Thanh Xuân |
| 4946813 | Số 1 đường Giải Phóng – phường Bạch Mai – ĐHBK | Hai Bà Trưng |
| 4946811 | 556 Nguyễn Văn Cừ | Long Biên |
| 6123215 | OceanPark | Gia Lâm |

# 3. Data Source 1 — Ground Measurements (OpenAQ v3 API)

Hourly observations crawled via OpenAQ v3 REST API with automatic retry logic (rate-limit handling 429/408/5xx). Data was collected in 14-day sliding windows to avoid API pagination limits.

| Parameter | Unit | Description |
| --- | --- | --- |
| PM2.5 | µg/m³ | Fine particulate matter — primary prediction target |
| CO | µg/m³ | Carbon monoxide |
| NO₂ | µg/m³ | Nitrogen dioxide |
| O₃ | µg/m³ | Ozone |
| SO₂ | µg/m³ | Sulfur dioxide |

Collection parameters:

Date range: 2024-01-29 → 2026-05-15 (≈ 837 days)

Frequency: hourly; daily averages computed downstream

API limit: 1,000 records per request; 14-day window batching

Output: all_stations_crawled.csv + 6 per-station CSV files

Combined dataset: all_stations_satellite_hourly.csv (merged with satellite)

# 4. Data Source 2 — Meteorological Data (Open-Meteo Archive API)

Hourly reanalysis weather data fetched from the Open-Meteo Archive API (free, no API key). Coordinates for each station were retrieved automatically from the OpenAQ API.

| Variable | Renamed Column | Unit |
| --- | --- | --- |
| temperature_2m | temperature_2m_C | °C |
| relative_humidity_2m | relative_humidity_pct | % |
| dew_point_2m | dew_point_2m_C | °C |
| wind_speed_10m | wind_speed_10m_kmh | km/h |
| wind_direction_10m | wind_direction_deg | ° |
| wind_gusts_10m | wind_gusts_10m_kmh | km/h |
| wind u-component | wind_u_10m | m/s |
| wind v-component | wind_v_10m | m/s |
| pressure_msl | pressure_msl_hPa | hPa |
| precipitation | precipitation_mm | mm |
| cloud_cover | cloud_cover_pct | % |
| boundary_layer_height | boundary_layer_height_m | m |
| visibility | visibility_m | m |

# 5. Data Source 3 — Satellite Observations (Sentinel-5P / Sentinel-2)

Satellite-derived atmospheric indices extracted per station using spatial buffer aggregation (pixels within radius of station coordinates). For each band, six summary statistics are computed: valid_pixels, mean, std, min, max, median.

| Band | Sensor | Description | Statistics |
| --- | --- | --- | --- |
| NO₂ | Sentinel-5P | Nitrogen dioxide column | 6 (mean, std, min, max, median, vpix) |
| CO | Sentinel-5P | Carbon monoxide column | 6 |
| SO₂ | Sentinel-5P | Sulfur dioxide column | 6 |
| AER-AI 340/380 | Sentinel-5P | Aerosol absorbing index | 6 |
| NDVI | Sentinel-2 | Vegetation index | 6 |
| NDBI | Sentinel-2 | Built-up index | 5 (mean, std, min, max, median) |
| NDWI | Sentinel-2 | Water index | 5 |

For the 3D CNN model, 30 features (5 bands × 6 stats) are arranged as a structured 6×5 grid:

3D Satellite Grid Layout
Rows (6): valid_pixels / mean / std / min / max / median
Cols (5): NO₂ / CO / SO₂ / Aerosol-AI / NDVI
→ Input tensor shape: (B, 1, 7-days, 6-stats, 5-bands)

# 6. Data Preprocessing

Key preprocessing steps applied in scripts/preprocess_satellite_pm25_daily.py and the load_daily() function in training scripts:

| Step | Method | Detail |
| --- | --- | --- |
| Date gap filling | Time-index reindex | Full daily calendar per station |
| PM2.5 interpolation | Linear (time) | Max 3 consecutive missing days |
| Satellite imputation | Linear + median fill | Per station, limit=10 days |
| Lag features | Pandas shift() | lag1, lag2, lag3, lag7, roll3, roll7, roll7std, delta |
| Seasonal encoding | sin/cos transform | sin_doy, cos_doy, sin_month, cos_month |
| Feature scaling | StandardScaler | Fit on train only; transform all splits |
| Target transform | log1p + StandardScaler | Applied to PM2.5 for neural training |
| PM2.5 filter | Range filter | Keep only 0 < PM2.5 ≤ 500 µg/m³ |

# 7. Dataset Split

A global chronological (70 / 15 / 15 %) split is applied across all 6 stations. Split boundaries are determined by calendar date (not per-station row count) to preserve temporal ordering.

| Split | Fraction | Rows (6 stations) | Period (approx.) |
| --- | --- | --- | --- |
| Train | 70 % | 1,109 | Jan 2024 → late 2025 |
| Val | 15 % | 492 | Autumn / Winter 2025 |
| Test | 15 % | 643 | Winter / Spring 2026 |

Val sub-splits for alpha selection:

Val-early (first 50% = 246 rows): autumn 2025

Val-late  (last 50%  = 246 rows): winter 2025 — temporally closest to test

# 8. Seasonal Distribution Shift (Core Challenge)

The most significant data challenge is a seasonal distribution shift between the validation and test periods:

Val period (autumn/winter 2025): mean PM2.5 ≈ 47 µg/m³, high values common (winter pollution season)

Test period (winter/spring 2026): mean PM2.5 ≈ 38 µg/m³, narrower range 8–132 µg/m³

Training range: 5–467 µg/m³ (extreme events present)

Impact: Neural models trained on winter val patterns systematically overestimate spring test values. Mitigation: linear calibration + val-late alpha selection (uses only the winter-proximate half of val).